Pioneer River Water LHS-1

Direct Step 0-7 Calculation and Detailed Code Explanation

BudgetE | Main control volume = Region 1 | 16,186 elements

Purpose. This report follows one compact Python script from the model files to the final Main-CV water-storage change. The emphasis is not on adding separate verification routines. Instead, each code block is explained in detail so that the origin, indexing, unit and physical meaning of every quantity used in the calculation are explicit.

Hi=di+ηi

He=H1+H2+H33

VCV=ΣVe,    ΔV=V(T₁)-V(T₀)

The direct calculation uses only hgrid.gr3, the Main-CV element list and out2d elevation/wet-dry output. zCoordinates is not used in the direct calculation; it was used only as an independent check of the bathymetry/elevation sign convention.

# 1. Files, run and saved states

RUN = Path( r"R:\Estuary_SCHISM-Q8213\9.0 Developed models\Pioneer" r"\Pioneer_17_waterlevel_flow_calibration_AprJun2025_BudgetE" ) HGRID = RUN / "hgrid.gr3" CV_FILE = RUN / "cv_pioneer_nested_M4_N1_elements.txt" OUT = RUN / "outputs" # Exact saved states used for the 30-day storage calculation. STATES = { "T0": (OUT / "out2d_1.nc", 0, 900.0), "T1": (OUT / "out2d_5.nc", 191, 2592000.0), } R_EARTH = 6371008.8 # m

RUN points to the completed Pioneer BudgetE simulation. HGRID is the static SCHISM horizontal grid. CV_FILE contains the element IDs that define the Main control volume. OUT is the directory containing the saved SCHISM outputs.

The calculation uses two already-established saved states. T0 is out2d_1.nc record 0 at 900 s. T1 is out2d_5.nc record 191 at 2,592,000 s. These are the beginning and end snapshots used for the 30-day storage change.

| Item | Source | What is used |
| --- | --- | --- |
| Main CV | cv_pioneer_nested_M4_N1_elements.txt | 16,186 element IDs |
| Static mesh | hgrid.gr3 | node longitude, latitude, bathymetric depth and element-node connectivity |
| T0 state | out2d_1.nc, record 0 | elevation and dryFlagElement at 900 s |
| T1 state | out2d_5.nc, record 191 | elevation and dryFlagElement at 2,592,000 s |

# 2. Step 0 - Read hgrid.gr3

def read_grid(): with HGRID.open("r", encoding="utf-8", errors="ignore") as f: title = f.readline().strip() ne, nn = map(int, f.readline().split()[:2]) lon = np.full(nn + 1, np.nan) lat = np.full(nn + 1, np.nan) depth = np.full(nn + 1, np.nan) # node ID -> bathymetric depth d [m] for _ in range(nn): p = f.readline().split() nid = int(p[0]) lon[nid] = float(p[1]) lat[nid] = float(p[2]) depth[nid] = float(p[3]) elems = [None] * (ne + 1) # element ID -> its node IDs for _ in range(ne): p = f.readline().split() eid, nv = int(p[0]), int(p[1]) elems[eid] = tuple(map(int, p[2:2 + nv])) return title, lon, lat, depth, elems

The first two lines of hgrid.gr3 give the grid title and the numbers of elements and nodes. The next NN lines are node records. For each node, the script reads: node ID, x coordinate, y coordinate and the fourth column d. In this Pioneer grid x and y are longitude and latitude in degrees, while d is bathymetric depth in metres.

The arrays lon, lat and depth are deliberately created with length NN+1. Index 0 is unused. This makes the Python index identical to the SCHISM node ID: depth[39107] means 'the depth stored for node 39107'.

After the node section, hgrid.gr3 contains the element connectivity. Each element record gives an element ID, the number of vertices, and the node IDs belonging to that element. The script stores this as elems[eid]. Therefore, for a triangular element, elems[eid] returns exactly three model node IDs.

| Object in code | Spatial basis | Unit | Meaning |
| --- | --- | --- | --- |
| lon[nid] | node | degree E | node longitude |
| lat[nid] | node | degree N | node latitude |
| depth[nid] | node | m | SCHISM bathymetric depth d; positive-down convention |
| elems[eid] | element to nodes | - | tuple of node IDs defining that element |

# 3. Step 0 - Read the Main-CV element IDs

def read_main_cv(): eids = [] for line in CV_FILE.read_text(encoding="utf-8", errors="ignore").splitlines(): s = line.strip() if s and not s.startswith("#"): eids.append(int(float(s.split()[0]))) return np.asarray(sorted(set(eids)), dtype=int)

The Main CV is represented as a list of SCHISM element IDs. The script reads the first value on each non-empty, non-comment line, removes duplicates and sorts the IDs. At this point the calculation has a set of element IDs, not node IDs. These element IDs are the objects that will be integrated.

The important mapping used later is therefore: Main-CV element ID -> hgrid connectivity -> the three node IDs belonging to that element.

| Actual walkthrough output A - mesh and Main-CV objects |  |
| --- | --- |
| Variable / check | Actual Pioneer value |
| Grid title | hgrid_deepen.gr3 |
| Mesh size | 130,884 elements; 67,762 nodes |
| lon / lat / depth array shape | (67,763,) each; index 0 intentionally unused |
| First node | lon[1] = 149.091928; lat[1] = -21.147282; depth[1] = 1.000000 m |
| Main-CV array | cv.shape = (16,186,); min eid = 149; max eid = 130,878 |
| First five Main-CV eids | [149, 150, 152, 153, 156] |
| Representative connectivity | elems[149] = (7917, 8016, 8107) |

# 4. Step 3 - Calculate the horizontal area of each Main-CV element

def metric_xy(lon, lat): lon0 = np.nanmean(lon[1:]) lat0 = np.nanmean(lat[1:]) x = R_EARTH * math.cos(math.radians(lat0)) * np.deg2rad(lon - lon0) y = R_EARTH * np.deg2rad(lat - lat0) return x, y def triangle_area(eid, elems, x, y): n1, n2, n3 = elems[eid] return 0.5 * abs( (x[n2] - x[n1]) * (y[n3] - y[n1]) - (x[n3] - x[n1]) * (y[n2] - y[n1]) )

The hgrid coordinates cannot be inserted directly into a planar area formula because they are longitude and latitude in degrees. metric_xy uses a local equirectangular approximation to convert every node to Cartesian x/y coordinates in metres. The longitudinal distance is multiplied by cos(lat0) to account for the convergence of meridians away from the equator. The conversion is centred on the mean longitude and latitude of the mesh, and REARTH = 6,371,008.8 m supplies the length scale.

triangle_area then starts from one element ID. elems[eid] returns n1, n2 and n3. The metric x/y coordinates of those three nodes are inserted into the standard triangle cross-product formula. Because x and y are metres, the result Ae is square metres.

Ae=12abs((x2-x1)(y3-y1)-(x3-x1)(y2-y1))

Area is static: it depends only on the horizontal mesh, not on T0 or T1. Therefore the script calculates one area for each of the 16,186 Main-CV elements once and reuses the same area at both saved states.

| Actual walkthrough output B - local coordinates and triangle area |  |
| --- | --- |
| Variable / check | Actual Pioneer value |
| Mesh mean longitude / latitude | lon0 = 149.204524109 deg; lat0 = -21.214878251 deg |
| cos(lat0) | 0.932229865114 |
| Element 149 nodes | (7917, 8016, 8107) |
| Node 7917 local x / y | -4,724.079970 m / +8,722.726001 m |
| Node 8016 local x / y | -4,708.116427 m / +8,703.378058 m |
| Node 8107 local x / y | -4,698.268787 m / +8,726.284244 m |
| Area of element 149 | 278.097754 m² |
| Main-CV total plan area | 4,575,495.046715 m² |

# 5. Read one out2d saved state

def read_state(path, record, expected_time, ne, nn): with Dataset(path) as ds: time = float(ds.variables["time"][record]) eta0 = np.ma.filled( ds.variables["elevation"][record, :], np.nan ).astype(float) dry0 = np.ma.filled( ds.variables["dryFlagElement"][record, :], np.nan ).astype(float) if abs(time - expected_time) > 2.0: raise RuntimeError(f"Unexpected saved time in {path.name}: {time}") # Convert NetCDF's 0-based arrays to 1-based arrays so model node/element # IDs can be used directly: eta[node_id], dry[element_id]. eta = np.full(nn + 1, np.nan) dry = np.full(ne + 1, np.nan) eta[1:] = eta0 dry[1:] = dry0 return time, eta, dry

For one saved state the script reads three things from out2d: time, elevation and dryFlagElement. elevation is node-based. dryFlagElement is element-based. This distinction is critical because the same element ID cannot be used directly to index a node variable.

The raw NetCDF arrays use Python/NetCDF zero-based positions, while SCHISM node and element IDs in hgrid start at 1. To make later indexing explicit, the script creates new arrays with one unused position at index 0. After eta[1:] = eta0, eta[node_id] is the elevation for that exact SCHISM node. After dry[1:] = dry0, dry[element_id] is the dry flag for that exact SCHISM element.

| Variable | Basis | Unit / values | Physical meaning |  |
| --- | --- | --- | --- | --- |
| elevation | node | m; positive upward | water-surface elevation η |  |
| dryFlagElement | element | 0 = wet; 1 = dry | whether the whole element is active as water at this saved state |  |
| time | saved record | s elapsed | model time of the selected snapshot |  |
| Actual walkthrough output C - saved-state arrays |  |  |  |
| Variable / check |  | Actual Pioneer value |  |
| T0 source |  | out2d_1.nc; record 0; time = 900 s |  |
| T0 elevation example |  | eta[1] = +0.002882711 m |  |
| T0 Main-CV dry state |  | 12,111 wet; 4,075 dry; total = 16,186 |  |
| T1 source |  | out2d_5.nc; record 191; time = 2,592,000 s |  |
| T1 elevation example |  | eta[1] = +0.957436442 m |  |
| T1 Main-CV dry state |  | 15,562 wet; 624 dry; total = 16,186 |  |
| Array indexing |  | eta.shape = (67,763,); dry.shape = (130,885,); index 0 = NaN/unused |  |

# 6. Steps 1, 2, 4, 5 and 6 - The element-by-element calculation

def main_cv_volume(cv, elems, depth, area, eta, dry): total = 0.0 wet_count = 0 dry_count = 0 # Start from an element ID in the Main CV. for eid in cv: # STEP 4: dryFlagElement is element-based. # 0 = wet; 1 = dry. if dry[eid] not in (0.0, 1.0): raise RuntimeError(f"Bad dryFlagElement at element {eid}: {dry[eid]}") D_e = int(dry[eid]) if D_e == 1: dry_count += 1 continue wet_count += 1 # Static hgrid connectivity gives: # element ID -> the three node IDs belonging to that triangle. n1, n2, n3 = elems[eid] # STEP 1: node water depth H_i = d_i + eta_i [m]. H1 = depth[n1] + eta[n1] H2 = depth[n2] + eta[n2] H3 = depth[n3] + eta[n3] # STEP 2: triangle mean water depth [m]. H_e = (H1 + H2 + H3) / 3.0 # STEP 5: wet-element water volume [m3]. V_e = area[eid] * H_e # STEP 6: discrete spatial integration over the Main CV. total += V_e return total, wet_count, dry_count

This function is the core of the calculation. The loop begins with one Main-CV element ID eid. The first time-dependent question is whether this element is wet or dry, so dry[eid] is read before any nodal water-depth calculation.

Step 4 - wet/dry decision. dryFlagElement is already element-based, so no node mapping is needed to answer this question. If dry[eid] = 1, the element contains no active water volume in this calculation. The script counts it as dry and immediately continues to the next element. No nodal elevation or water depth is calculated for that dry element.

If dry[eid] = 0, the element is wet. Only then does the script use elems[eid] to retrieve n1, n2 and n3, the three node IDs that define this triangle. This is the exact element-to-node connection in the model grid.

Step 1 - node water depth. depth[n1] and eta[n1] refer to the bathymetric depth and free-surface elevation at the same model node n1. The same operation is repeated for n2 and n3.

Hi=di+ηi

Here di is the bathymetric depth read from hgrid.gr3 and ηi is the free-surface elevation read from out2d. For this Pioneer run, the sign convention has been independently confirmed from zCoordinates: zbottom = -di and zsurface = ηi. Therefore Hi = zsurface - zbottom = ηi - (-di) = di + ηi. Hi is the physical nodal water-column height in metres.

Step 2 - node to element. The three nodal water-column heights are averaged to obtain one mean water depth He for the triangular element.

He=H1+H2+H33

Step 5 - element water volume. For a wet element, Ae is in m² and He is in m, so their product is the element water volume in m³. Dry elements have already been skipped, which is computationally equivalent to multiplying by (1 - De).

Ve=(1-De)AeHe

Step 6 - Main-CV integration. total starts at zero. Each wet element volume Ve is added once. When the loop has visited every Main-CV element, total is the discrete horizontal integral of water volume over the complete Main CV.

VCV=ΣVe

| Code operation | Starts from | Returns / uses |  |
| --- | --- | --- | --- |
| De = dry[eid] | element ID | element wet/dry state |  |
| n1,n2,n3 = elems[eid] | wet element ID | three node IDs |  |
| depth[n] + eta[n] | node ID | node water depth Hi [m] |  |
| (H1+H2+H3)/3 | three node depths | element mean depth He [m] |  |
| area[eid] * He | element area + mean depth | element volume Ve [m³] |  |
| total += Ve | each wet element volume | Main-CV volume VCV [m³] |  |
| Actual walkthrough output D - representative wet element 149 |  |  |
| Quantity | T0 | T1 |
| dry[149] | 0 (wet) | 0 (wet) |
| d at nodes 7917 / 8016 / 8107 [m] | +0.637169930 / +0.740344440 / +0.295883010 | +0.637169930 / +0.740344440 / +0.295883010 |
| η at nodes 7917 / 8016 / 8107 [m] | -0.000025774 / -0.000023432 / -0.000024063 | +1.570728660 / +1.569965243 / +1.569500685 |
| H1 / H2 / H3 = d + η [m] | +0.637144156 / +0.740321008 / +0.295858947 | +2.207898590 / +2.310309683 / +1.865383695 |
| He [m] | +0.557774704 | +2.127863989 |
| A149 [m2] | 278.097754 | 278.097754 |
| V149 = A149 He [m3] | +155.115893 | +591.754196 |

# 7. Step 7 - Calculate T0, T1 and the storage change

def main(): title, lon, lat, depth, elems = read_grid() cv = read_main_cv() ne, nn = len(elems) - 1, len(depth) - 1 # Step 3 is time-independent: calculate Main-CV element areas once. x, y = metric_xy(lon, lat) area = np.zeros(ne + 1) for eid in cv: if len(elems[eid]) != 3: raise RuntimeError(f"Element {eid} is not a triangle.") area[eid] = triangle_area(eid, elems, x, y) results = {} for name, (path, record, expected_time) in STATES.items(): time, eta, dry = read_state(path, record, expected_time, ne, nn) V, nwet, ndry = main_cv_volume(cv, elems, depth, area, eta, dry) results[name] = V print(f"{name}: time={time:.0f} s") print(f" wet elements = {nwet:,}") print(f" dry elements = {ndry:,}") print(f" Main-CV volume = {V:,.3f} m3") dV = results["T1"] - results["T0"] print() print(f"hgrid title = {title}") print(f"Main-CV elements = {len(cv):,}") print(f"Main-CV area = {np.sum(area[cv]):,.3f} m2") print(f"V(T0) = {results['T0']:,.3f} m3") print(f"V(T1) = {results['T1']:,.3f} m3") print(f"DeltaV = V1 - V0 = {dV:+,.3f} m3") if __name__ == "__main__": main()

main first reads the static grid and the Main-CV element list. It converts the mesh to metric coordinates and calculates all Main-CV triangle areas once. It then repeats the same volume function for T0 and T1.

The final Water LHS storage change is the final Main-CV inventory minus the initial Main-CV inventory. A positive ΔV means that the Main CV contains more water at T1 than at T0.

ΔV=V(T₁)-V(T₀)

# 8. Numerical result for the 30-day Pioneer Main CV

The numerical values below use the same local equirectangular geometry as Step 3 with REARTH = 6,371,008.8 m, matching the current production water-budget script 110. The first walkthrough execution used 6,371,000.0 m and returned ΔV = 7,201,230.547 m³. Under this projection x and y scale linearly with R, so area and every AeHe volume scale exactly with R²; bathymetry, elevation, dry flags and water-column heights are unchanged. Harmonising the radius therefore gives the values shown below.

| Quantity | Result | Meaning |
| --- | --- | --- |
| V(T0) | 5,240,571.546 m3 | Main-CV water inventory at 900 s |
| V(T1) | 12,441,821.987 m3 | Main-CV water inventory at 2,592,000 s |
| ΔV = V(T1) - V(T0) | +7,201,250.441 m3 | 30-day increase in Main-CV water storage |

Therefore the Water LHS for the selected 30-day interval is ΔV = +7,201,250.441 m3, or approximately +7.20 × 106 m3. The positive sign indicates an increase in stored water within the Main CV.

Walkthrough interpretation. The added boxes show the actual contents of the key variables after each function, so the calculation can be followed from model IDs and node coordinates to one real element volume and finally to the Main-CV storage change.