# Water Budget RHS Method B - Step-by-Step hand check

# 1. Purpose

This verification checks the Water RHS Method B calculation in the same order as Section 2.3 of the budget methodology. The objective is to demonstrate that the Python implementation correctly converts nodal velocity and vertical geometry into water transport across one selected SCHISM-style edge.

# 2. Verification configuration

Figure 1 uses the same triangular-grid concept as the Water Method B schematic. One internal vertical edge is selected. The Main CV is defined on the right-hand side of this edge, so positive flux points to the right. Both elements adjacent to the selected internal edge are assumed to remain wet throughout the verification.

SCHISM-style verification mesh. The thick vertical line is the selected internal edge.

The prescribed values are:

| Quantity | Prescribed value |
| --- | --- |
| Node i | xi,yi=1,0 m |
| Node j | xj,yj=1,1 m |
| Nodal velocity | u=2, v=1 m/s |
| Vertical levels | z=-3,-2,-1,0 m |
| Number of physical intervals | 3 |
| Verification duration | 3600 s |

The exact cross-edge velocity component is un=2 m/s, the edge length is 1 m and the total water depth is 3 m. Therefore the independent analytical discharge is

Qexact=unLsH=2×1×3=6 m3 s-1.

For a constant one-hour period,

ΔVexact=QexactΔt=6×3600=21600 m3.

# 3. Step-by-step verification of Water RHS Method B

## Step 1 - Read nodal velocity at each vertical level

At nodes i and j,

ui,k,vi,k=2,1 m s-1,

uj,k,vj,k=2,1 m s-1.

The same values are prescribed at all four saved vertical levels. This removes vertical interpolation ambiguity from the analytical verification.

Python result: all prescribed nodal velocities are recovered exactly. Status: PASS.

## Step 2 - Calculate edge length

The Method B edge length is

Ls=xj-xi2+yj-yi2.

Therefore,

Ls=1-12+1-02=1 m.

Python result: Ls=1 m. Expected: 1 m. Status: PASS.

## Step 3 - Define the unit normal to the edge

The documented unit normal is

nx=yj-yiLs,

ny=xi-xjLs.

With node order i=1,0 to j=1,1,

nx,ny=1,0.

The vector therefore points to the right, which is defined as into the Main CV for this test. Its magnitude is

nx2+ny2=1.

Python result: nx,ny=1,0. Status: PASS.

## Step 4 - Project nodal velocity onto the edge-normal direction

At node i,

un,i,k=ui,knx+vi,kny.

At node j,

un,j,k=uj,knx+vj,kny.

The edge value is

un,s,k=un,i,k+un,j,k2.

Substitution gives

un,i,k=21+10=2 m s-1,

un,j,k=21+10=2 m s-1,

un,s,k=2+22=2 m s-1.

Python result: un,s,k=2 m/s. Status: PASS.

## Step 5 - Calculate layer thickness at the two edge nodes

For each vertical interval,

hi,k=zi,k+1-zi,k,

hj,k=zj,k+1-zj,k.

The edge thickness is

hs,k=hi,k+hj,k2.

For z=-3,-2,-1,0 m,

hi,k=hj,k=hs,k=1 m

for each of the three intervals. The total edge depth is

k​hs,k=3 m.

Python result: three 1 m intervals and total depth 3 m. Status: PASS.

## Step 6 - Calculate the cross-sectional area of one edge layer

For each edge layer,

As,ksection=Lshs,k.

Therefore,

As,ksection=1×1=1 m2.

The three-layer total cross-sectional area is 3 m2.

Python result: 1 m2 per layer. Status: PASS.

## Step 7 - Calculate water flux through one edge layer

The Method B edge-layer flux is

qs,kt=Wst un,s,kt As,ksectiont.

For each layer,

qs,k=1×2×1=2 m3 s-1.

The unit chain is

m s-1 m2=m3 s-1.

Python result: 2 m3/s for each layer. Status: PASS.

## Step 8 - Sum all three vertical layers of one edge

The total selected-edge discharge is

Qst=k​qs,kt.

Therefore,

Qs=2+2+2=6 m3 s-1.

This is identical to the independent analytical value Qexact=6 m3/s.

Python result: Qs=6 m3/s. Status: PASS.

## Step 9 - Sum all selected boundary edges

For this verification, the selected Method B boundary contains exactly one edge. Therefore,

Qnett=Qst=6 m3 s-1.

Python result: Qnet=6 m3/s. Status: PASS.

## Step 10 - Integrate over time

The Method B cumulative volume is

ΔVRHS,B=tbegintendQnett dt.

The Python program uses the trapezoidal rule. For constant Qnet=6 m3/s over 3600 s,

ΔVRHS,B=6×3600=21600 m3.

Python result: 21600 m3. Expected: 21600 m3. Status: PASS.

# 4. Independent analytical edge-flux check

The exact flux through the selected rectangular vertical section can be calculated without the Method B reconstruction:

Qexact=A​u⋅n dA.

Because u · n = 2 m/s is uniform and A = 1 × 3 = 3 m²,

Qexact=2×3=6 m3 s-1.

The Method B result is therefore exactly equal to the independently derived analytical flux.

# 5. Verification summary

| Verification item | Calculated | Expected | Status |
| --- | --- | --- | --- |
| Edge length | 1 m | 1 m | PASS |
| Unit normal | (1, 0) | (1, 0) | PASS |
| Edge-normal velocity | 2 m/s | 2 m/s | PASS |
| Layer thickness | 1 m per layer | 1 m per layer | PASS |
| Layer section area | 1 m² | 1 m² | PASS |
| Layer flux | 2 m³/s | 2 m³/s | PASS |
| Edge flux | 6 m³/s | 6 m³/s | PASS |
| One-edge net flux | 6 m³/s | 6 m³/s | PASS |
| One-hour cumulative volume | 21600 m³ | 21600 m³ | PASS |

All reported single-edge analytical verification checks completed with PASS status.