Pioneer River Water

Direct Main-Control-Volume Water-Storage Calculation

Method definition, data lineage, equations, variables and code traceability

Point sources | Main control volume: Region 1 | 16,186 elements

# 1. LHS-1 summary

Purpose. LHS-1 quantifies the change in total water stored inside the Pioneer River Main control volume between two selected SCHISM saved states. The calculation is a direct inventory calculation: it reconstructs the water volume of every wet Main-CV element at T0 and T1 and subtracts the initial inventory from the final inventory.

Calculation scope. The direct calculation uses hgrid.gr3, the Main-CV element list, and out2d elevation and dryFlagElement output. zCoordinates is not used to calculate LHS-1; it is used only as an independent sign-convention check for bathymetric depth and water-surface elevation.

The formal definition of LHS-1 is:

| ΔV = VCV(T1) - VCV(T0) | (1) |
| --- | --- |

A positive ΔV means that the Main control volume contains more water at T1 than at T0. For the selected 30-day interval, the updated SCHISM-native-area result is +7,221,292.911 m³.

# 2. Input data and variable register

Table 1. Input files and selected saved states used by the LHS-1 calculation

| Item | Source file / output | Quantity used | Selected value |
| --- | --- | --- | --- |
| Static mesh | hgrid.gr3 | Node horizontal coordinates; bathymetric depth d; element-node connectivity | Static |
| Main control volume | cv_pioneer_nested_M4_N1_elements.txt | 16,186 SCHISM element IDs defining Region 1 | Static |
| Initial state T0 | out2d_1.nc, record 0 | elevation; dryFlagElement; model time = 900 s | 900 s |
| Final state T1 | out2d_5.nc, record 191 | elevation; dryFlagElement; model time = 2,592,000 s | 2,592,000 s |
| Runtime horizontal geometry | param.nml | ics; rearth_eq; rearth_pole | 2; 6,378,206.4 m; 6,378,206.4 m |

Source: BudgetE model directory and SCHISM output files listed in the Python configuration block.

Table 2. Definition, unit, source and value/status of variables used in the LHS-1 equations

| Symbol | Python variable | Meaning | Basis | Unit | Source / definition | Value / status |
| --- | --- | --- | --- | --- | --- | --- |
| ΔV | results["DeltaV"] | LHS-1 water-storage change | CV | m³ | Eq. (1); calculated | +7,221,292.911 |
| V_CV | total / results[state] | Total water inventory in Main CV | CV | m³ | Eq. (8); calculated | varies by state |
| T0, T1 | STATES | Selected initial/final saved states | time | s | out2d_1.nc / out2d_5.nc | 900 / 2,592,000 |
| e | eid | SCHISM element index | element | - | hgrid.gr3 / CV file | Main-CV IDs |
| i | nid | SCHISM node index | node | - | hgrid.gr3 | 1 to 67,762 |
| n1,n2,n3 | elems[eid] | Three node IDs defining a triangular element | element→node | - | hgrid.gr3 | varies |
| λ_i | lon[nid] | Node longitude coordinate used by current geometry | node | degree | hgrid.gr3 | varies |
| R_eq, R_pole | rearth_eq, rearth_pole | SCHISM Earth-radius settings used by ics=2 | constant | m | param.nml | 6,378,206.4 / 6,378,206.4 |
| ics | ics | SCHISM horizontal-coordinate option | constant | - | param.nml | 2 |
| r_i | xnd[nid], ynd[nid], znd[nid] | Earth-centred node position used by SCHISM | node | m | Eq. (2); calculated | varies |
| xel.j, yel.j | x[nid], y[nid] | Local zonal/east and meridional/north coordinates of node jrelative to the element centre. | node | m | Eqs. (2)-(4); SCHISM ics=2 geometry | varies |
| A_e | area[eid] | Horizontal plan area of element e | element | m² | Eq. (4); hgrid geometry | varies |
| D_e | dry[eid] | Element wet/dry status | element | - | out2d dryFlagElement | 0 wet; 1 dry |
| d_i | depth[nid] | Bathymetric depth, positive-down | node | m | hgrid.gr3 | varies |
| η_i | eta[nid] | Water-surface elevation, positive-up | node | m | out2d elevation | varies |
| H_i | H1/H2/H3 | Nodal water-column height | node | m | Eq. (5); calculated | varies |
| H_e | H_e | Mean water depth of element e | element | m | Eq. (6); calculated | varies |
| V_e | V_e | Water volume of wet element e | element | m³ | Eq. (7); calculated | varies |

Source: hgrid.gr3; cv_pioneer_nested_M4_N1_elements.txt; out2d_1.nc; out2d_5.nc; and calculated quantities defined by Eqs. (1)-(8).

# 3. Calculation procedure and code traceability

The report and the Python script use the same Step 1-Step 10 numbering. The order below follows the actual execution sequence; there are no repeated Step 0 labels and no out-of-order Step 4 → Step 1 transitions.

Table 3. Ordered LHS-1 workflow used consistently in the report and Python script

| Step | Operation | Purpose |
| --- | --- | --- |
| Step 1 | Define input files and selected saved states | Files/records are explicit before calculation begins |
| Step 2 | Read hgrid.gr3 | Create 1-based node arrays and element connectivity |
| Step 3 | Read Main-CV element IDs | Define the elements to be integrated |
| Step 4 | Calculate SCHISM-native horizontal element areas | Read runtime geometry; Earth-centred XYZ → element-local xel/yel → signa() |
| Step 5 | Read one out2d saved state | Read time, node elevation and element wet/dry status |
| Step 6 | Apply wet/dry status | Exclude dry elements before water-volume calculation |
| Step 7 | Calculate nodal water-column depth | Map element → nodes and calculate H_i |
| Step 8 | Calculate element mean water depth | Average H1, H2 and H3 to H_e |
| Step 9 | Calculate element volume and Main-CV inventory | Calculate V_e and sum to V_CV |
| Step 10 | Calculate LHS-1 | Repeat T0/T1 inventory and calculate ΔV |

## Step 1 - Define input files and selected saved states

All external files and selected records are defined explicitly at the start of the script. This removes hidden file dependencies from later functions and makes the complete data lineage visible before any calculation is executed.

RUN = Path( r"R:\Estuary_SCHISM-Q8213" r"\9.0 Developed models" r"\Pioneer" r"\Pioneer_17_waterlevel_flow_calibration_AprJun2025_BudgetE" ) HGRID = RUN / "hgrid.gr3" PARAM = RUN / "param.nml" CV_FILE = RUN / "cv_pioneer_nested_M4_N1_elements.txt" OUT = RUN / "outputs" STATES = { "T0": (OUT / "out2d_1.nc", 0, 900.0), "T1": (OUT / "out2d_5.nc", 191, 2592000.0), }

## Step 2 - Read the SCHISM horizontal grid

The function now receives hgrid_path explicitly as an input argument. It reads the grid title, element/node counts, node horizontal coordinates, node bathymetric depth and element-to-node connectivity. Node and element arrays are stored with an unused index 0 so that Python indices match the original 1-based SCHISM model IDs.

def read_grid(hgrid_path): """ Read the SCHISM horizontal grid. Parameters ---------- hgrid_path : pathlib.Path Path to hgrid.gr3. """ with hgrid_path.open("r", encoding="utf-8", errors="ignore") as f: title = f.readline().strip() ne, nn = map(int, f.readline().split()[:2]) lon = np.full(nn + 1, np.nan) lat = np.full(nn + 1, np.nan) depth = np.full(nn + 1, np.nan) for _ in range(nn): p = f.readline().split() nid = int(p[0]) lon[nid] = float(p[1]) lat[nid] = float(p[2]) depth[nid] = float(p[3]) elems = [None] * (ne + 1) for _ in range(ne): p = f.readline().split() eid = int(p[0]) nv = int(p[1]) elems[eid] = tuple(map(int, p[2:2 + nv])) return title, lon, lat, depth, elems

Table 4. Actual mesh objects returned by Step 2

| Variable / check | Actual Pioneer value |
| --- | --- |
| Grid title | hgrid_deepen.gr3 |
| Mesh size | 130,884 elements; 67,762 nodes |
| lon / lat / depth shape | (67,763,) each; index 0 intentionally unused |
| First node | lon[1] = 149.091928; lat[1] = -21.147282; depth[1] = 1.000000 m |
| Representative connectivity | elems[149] = (7917, 8016, 8107) |

Source: hgrid.gr3 in the Pioneer BudgetE run directory.

## Step 3 - Read the Main-CV element IDs

The function receives cv_file explicitly. It reads the first value from each non-empty, non-comment line, removes duplicates and sorts the resulting SCHISM element IDs.

def read_main_cv(cv_file): """Read the Main control-volume SCHISM element IDs.""" eids = [] for line in cv_file.read_text( encoding="utf-8", errors="ignore" ).splitlines(): s = line.strip() if s and not s.startswith("#"): eids.append(int(float(s.split()[0]))) return np.asarray(sorted(set(eids)), dtype=int)

Table 5. Actual Main-CV element list returned by Step 3

| Variable / check | Actual Pioneer value |
| --- | --- |
| Number of Main-CV elements | 16,186 |
| Minimum / maximum element ID | 149 / 130,878 |
| First five element IDs | [149, 150, 152, 153, 156] |
| Element 149 connectivity | (7917, 8016, 8107) |

Source: cv_pioneer_nested_M4_N1_elements.txt, with element connectivity retrieved from hgrid.gr3.

## Step 4 - Calculate static horizontal element areas

The updated LHS-1 calculation reproduces the SCHISM horizontal geometry used at runtime. For ics = 2, hgrid x/y are longitude/latitude in degrees. SCHISM first maps every node to Earth-centred coordinates using rearth_eq and rearth_pole, then calculates an element centre, constructs an element-local east/north frame, projects the three element nodes to xel/yel, and evaluates the same signa() planar-triangle area used internally by SCHISM.

The corresponding SCHISM-native geometry equations used in the Python reproduction are:

| ri=Reqcosϕicosλi, Reqcosϕisinλi, Rpolesinϕi | (2) |
| --- | --- |
| xel,j=rj-rc⋅eE, yel,j=rj-rc⋅eN | (3) |

In Eq. (2), r_i is the Earth-centred node position. For each triangular element, SCHISM averages the three node positions to obtain the element centre r_c, derives the local east and north unit vectors from the centre longitude/latitude, and uses Eq. (3) to obtain xel and yel. Eq. (4) is the same signed-area expression used by SCHISM; the positive plan area is used in the storage calculation.

| Ae=12xel,1-xel,3yel,2-yel,3-xel,2-xel,3yel,1-yel,3 | (4) |
| --- | --- |

def read_param_numeric(text, key): """Read one numeric assignment from param.nml text.""" pattern = re.compile( rf"(?im)^\s*{re.escape(key)}\s*=\s*" rf"([+-]?(?:\d+(?:\.\d*)?¦\.\d+)(?:[eEdD][+-]?\d+)?)" ) match = pattern.search(text) if match is None: raise RuntimeError(f"Cannot find '{key}' in param.nml text.") return float(match.group(1).replace("D", "E").replace("d", "e")) def read_schism_geometry_settings(param_path): """ Read the actual SCHISM horizontal-coordinate settings used by this run. This reproduction is specifically for the present Pioneer configuration: ics = 2 rearth_eq == rearth_pole """ if not param_path.exists(): raise RuntimeError(f"Missing param.nml: {param_path}") text = param_path.read_text(encoding="utf-8", errors="ignore") ics = int(round(read_param_numeric(text, "ics"))) rearth_eq = read_param_numeric(text, "rearth_eq") rearth_pole = read_param_numeric(text, "rearth_pole") if ics != 2: raise RuntimeError( f"This script reproduces SCHISM ics=2 lon/lat geometry, but ics={ics}." ) if abs(rearth_eq - rearth_pole) > 1.0e-6: raise RuntimeError( "This exact Pioneer reproduction currently expects " "rearth_eq == rearth_pole.\n" f"rearth_eq={rearth_eq}, rearth_pole={rearth_pole}" ) return ics, rearth_eq, rearth_pole def earth_centred_xyz(lon, lat, rearth_eq, rearth_pole): """ Reproduce the SCHISM ics=2 node mapping from lon/lat [degree] to Earth-centred xnd/ynd/znd [m]. """ lam = np.deg2rad(lon) phi = np.deg2rad(lat) xnd = rearth_eq * np.cos(phi) * np.cos(lam) ynd = rearth_eq * np.cos(phi) * np.sin(lam) znd = rearth_pole * np.sin(phi) return xnd, ynd, znd def schism_native_triangle_area( eid, elems, xnd, ynd, znd, return_details=False, ): """ Calculate one triangular element area in the SCHISM ics=2 pathway. node lon/lat -> Earth-centred XYZ -> element-centre XYZ -> element-local east/north frame -> xel/yel -> SCHISM signa() """ n1, n2, n3 = elems[eid] nodes = np.asarray([n1, n2, n3], dtype=int) xc = float(np.mean(xnd[nodes])) yc = float(np.mean(ynd[nodes])) zc = float(np.mean(znd[nodes])) lon_c = math.atan2(yc, xc) lat_c = math.atan2(zc, math.hypot(xc, yc)) east = np.asarray([ -math.sin(lon_c), +math.cos(lon_c), 0.0, ], dtype=float) north = np.asarray([ -math.sin(lat_c) * math.cos(lon_c), -math.sin(lat_c) * math.sin(lon_c), +math.cos(lat_c), ], dtype=float) north_norm = float(np.linalg.norm(north)) if north_norm == 0.0: raise RuntimeError( f"Element {eid}: zero-length local north axis." ) north = north / north_norm centre = np.asarray([xc, yc, zc], dtype=float) xel = np.zeros(3, dtype=float) yel = np.zeros(3, dtype=float) for j, nid in enumerate(nodes): delta = np.asarray([ xnd[nid], ynd[nid], znd[nid], ], dtype=float) - centre xel[j] = float(np.dot(delta, east)) yel[j] = float(np.dot(delta, north)) signed_area = 0.5 * ( (xel[0] - xel[2]) * (yel[1] - yel[2]) - (xel[1] - xel[2]) * (yel[0] - yel[2]) ) if signed_area <= 0.0: raise RuntimeError( f"Element {eid} has non-positive SCHISM signa() area: " f"{signed_area:.16e} m2" ) area = float(signed_area) if return_details: return area, { "nodes": tuple(int(v) for v in nodes), "lon_c_deg": math.degrees(lon_c), "lat_c_deg": math.degrees(lat_c), "xel": xel, "yel": yel, "signed_area": signed_area, } return area def build_main_cv_native_areas( cv, elems, xnd, ynd, znd, ): """Calculate SCHISM-native horizontal areas for all Main-CV elements.""" ne = len(elems) - 1 area = np.zeros(ne + 1, dtype=float) for eid in cv: if elems[eid] is None: raise RuntimeError( f"Element {eid} is missing from hgrid connectivity." ) if len(elems[eid]) != 3: raise RuntimeError( f"Element {eid} is not a triangle." ) area[eid] = schism_native_triangle_area( eid=eid, elems=elems, xnd=xnd, ynd=ynd, znd=znd, ) return area

Table 6. Actual geometry walkthrough for representative element 149

| Variable / check | Actual Pioneer value |
| --- | --- |
| SCHISM horizontal-coordinate option | ics = 2 |
| Runtime Earth radii | rearth_eq = 6,378,206.4 m; rearth_pole = 6,378,206.4 m |
| Element 149 nodes | (7917, 8016, 8107) |
| Element 149 centre lon / lat | 149.159085333334° / -21.136480333364° |
| Element 149 local coordinates | 7917: xel=-13.948036756335, yel=+5.269177440395 m; 8016: +2.042017806026, -14.100618947086 m; 8107: +11.906018949676, +8.831441506088 m |
| Area A149 | 278.874295796000 m² |
| Main-CV total plan area | 4,588,208.795705 m² |
| Change from previous local-area method | +12,713.748990 m² (+0.28%) |

## Step 5 - Read one saved SCHISM state

For each selected state, the function receives the NetCDF path, record index, expected time, total element count and total node count as explicit inputs. elevation is node-based, whereas dryFlagElement is element-based. The raw 0-based NetCDF arrays are copied into 1-based arrays so that eta[node_id] and dry[element_id] use the exact SCHISM model IDs.

def read_state(path, record, expected_time, ne, nn): """ Read time, node elevation and element wet/dry flag for one saved state. """ with Dataset(path) as ds: time = float(ds.variables["time"][record]) eta0 = np.ma.filled( ds.variables["elevation"][record, :], np.nan, ).astype(float) dry0 = np.ma.filled( ds.variables["dryFlagElement"][record, :], np.nan, ).astype(float) if abs(time - expected_time) > 2.0: raise RuntimeError( f"Unexpected saved time in {path.name}: " f"read {time} s, expected {expected_time} s" ) eta = np.full(nn + 1, np.nan) dry = np.full(ne + 1, np.nan) eta[1:] = eta0 dry[1:] = dry0 return time, eta, dry

Table 7. Actual saved-state arrays read in Step 5

| Variable / check | Actual Pioneer value |
| --- | --- |
| T0 source | out2d_1.nc; record 0; time = 900 s |
| T0 elevation example | eta[1] = +0.002882711 m |
| T0 Main-CV wet/dry state | 12,111 wet; 4,075 dry; total = 16,186 |
| T1 source | out2d_5.nc; record 191; time = 2,592,000 s |
| T1 elevation example | eta[1] = +0.957436442 m |
| T1 Main-CV wet/dry state | 15,562 wet; 624 dry; total = 16,186 |
| Array indexing | eta.shape = (67,763,); dry.shape = (130,885,); index 0 unused |

Source: out2d_1.nc record 0; out2d_5.nc record 191; Main-CV membership from cv_pioneer_nested_M4_N1_elements.txt.

## Steps 6-9 - Element-by-element water-inventory calculation

Steps 6-9 are executed in one element loop because they operate on the same Main-CV element. The comments in the revised Python code follow the same order as the report: wet/dry decision → element-to-node mapping and nodal water depth → element mean depth → element volume and Main-CV integration.

### Step 6 - Apply the element-based wet/dry status

dryFlagElement is already element-based. If D_e = 1, the element is dry and is skipped before any nodal depth or volume is calculated. If D_e = 0, the element is wet and continues through Steps 7-9. This is computationally equivalent to the factor (1 - D_e) in Eq. (7).

### Step 7 - Calculate nodal water-column depth

For each wet triangular element, hgrid.gr3 connectivity provides the three node IDs n1, n2 and n3. At each node, bathymetric depth d_i is positive-down and free-surface elevation η_i is positive-up. Their sum therefore gives the physical water-column height:

| Hi = di + ηi | (5) |
| --- | --- |

The sign convention was independently checked against SCHISM zCoordinates output; zCoordinates is not used as an LHS-1 input.

### Step 8 - Calculate element mean water depth

The three nodal water-column heights are averaged to obtain one representative water depth for the triangular element:

| He = H1 + H2 + H33 | (6) |
| --- | --- |

### Step 9 - Calculate element volume and integrate the Main-CV inventory

For a wet element, horizontal plan area A_e [m²] is multiplied by element mean water depth H_e [m] to obtain element water volume V_e [m³]. Including the wet/dry indicator explicitly gives:

| Ve = (1 - De)AeHe | (7) |
| --- | --- |

The Main-CV inventory is then the discrete sum of all element volumes:

| VCV = ∑e ∈ CV Ve | (8) |
| --- | --- |

def main_cv_volume( cv, elems, depth, area, eta, dry, ): """Calculate total Main-CV water volume for one saved state.""" total = 0.0 wet_count = 0 dry_count = 0 for eid in cv: if dry[eid] not in (0.0, 1.0): raise RuntimeError( f"Bad dryFlagElement at element {eid}: {dry[eid]}" ) D_e = int(dry[eid]) if D_e == 1: dry_count += 1 continue wet_count += 1 n1, n2, n3 = elems[eid] H1 = depth[n1] + eta[n1] H2 = depth[n2] + eta[n2] H3 = depth[n3] + eta[n3] H_e = (H1 + H2 + H3) / 3.0 V_e = area[eid] * H_e total += V_e return total, wet_count, dry_count def print_element_state_diagnostic( eid, elems, depth, area, eta, dry, label, ): """Print the exact element-level quantities used in the LHS-1 inventory.""" n1, n2, n3 = elems[eid] D_e = int(dry[eid]) H1 = depth[n1] + eta[n1] H2 = depth[n2] + eta[n2] H3 = depth[n3] + eta[n3] H_e = (H1 + H2 + H3) / 3.0 V_e = 0.0 if D_e == 1 else area[eid] * H_e print(f"Element {eid} diagnostic at {label}") print(f" dryFlagElement = {D_e}") print(f" nodes = ({n1}, {n2}, {n3})") print( f" depth [m] = {depth[n1]:+.9f}, " f"{depth[n2]:+.9f}, {depth[n3]:+.9f}" ) print( f" eta [m] = {eta[n1]:+.9f}, " f"{eta[n2]:+.9f}, {eta[n3]:+.9f}" ) print( f" H1/H2/H3 [m] = {H1:+.9f}, {H2:+.9f}, {H3:+.9f}" ) print(f" H_e [m] = {H_e:+.9f}") print(f" A_e [m2] = {area[eid]:.12f}") print(f" V_e [m3] = {V_e:+.9f}") print()

Table 8. Representative element-149 calculation at T0 and T1

| Quantity | T0 | T1 |
| --- | --- | --- |
| dry[149] | 0 (wet) | 0 (wet) |
| d at nodes 7917 / 8016 / 8107 [m] | +0.637169930 / +0.740344440 / +0.295883010 | +0.637169930 / +0.740344440 / +0.295883010 |
| η at nodes 7917 / 8016 / 8107 [m] | -0.000025774 / -0.000023432 / -0.000024063 | +1.570728660 / +1.569965243 / +1.569500685 |
| H1 / H2 / H3 [m] | +0.637144156 / +0.740321008 / +0.295858947 | +2.207898590 / +2.310309683 / +1.865383695 |
| H_e [m] | +0.557774704 | +2.127863989 |
| A_149 [m²] | 278.8742958 | 278.8742958 |
| V_149 [m³] | +155.549028 | +593.404066 |

Source: hgrid.gr3; out2d_1.nc record 0; out2d_5.nc record 191; Eqs. (5)-(7).

## Step 10 - Calculate T0, T1 and the LHS-1 storage change

The top-level calculation function now also receives all major inputs explicitly. It reads the static grid and Main-CV definition, calculates static element areas once, calculates the Main-CV water inventory for each saved state.

def calculate_lhs1( hgrid_path, param_path, cv_file, states, rep_eid, ): """ Run the complete LHS-1 calculation using explicit input arguments. """ title, lon, lat, depth, elems = read_grid( hgrid_path=hgrid_path ) cv = read_main_cv( cv_file=cv_file ) ne = len(elems) - 1 nn = len(depth) - 1 ics, rearth_eq, rearth_pole = read_schism_geometry_settings( param_path=param_path ) xnd, ynd, znd = earth_centred_xyz( lon=lon, lat=lat, rearth_eq=rearth_eq, rearth_pole=rearth_pole, ) area = build_main_cv_native_areas( cv=cv, elems=elems, xnd=xnd, ynd=ynd, znd=znd, ) if rep_eid in set(cv.tolist()): rep_area, rep = schism_native_triangle_area( eid=rep_eid, elems=elems, xnd=xnd, ynd=ynd, znd=znd, return_details=True, ) print("SCHISM horizontal geometry") print(f" ics = {ics}") print(f" rearth_eq = {rearth_eq:.6f} m") print(f" rearth_pole = {rearth_pole:.6f} m") print(f" element {rep_eid} = nodes {rep['nodes']}") print( f" centre lon/lat = " f"{rep['lon_c_deg']:.12f}, {rep['lat_c_deg']:.12f} deg" ) print(f" signed area = {rep['signed_area']:.12f} m2") for j, nid in enumerate(rep["nodes"]): print( f" node {nid}: xel={rep['xel'][j]:+.12f} m, " f"yel={rep['yel'][j]:+.12f} m" ) print(f" area({rep_eid}) = {rep_area:.12f} m2") print() results = {} for name, (path, record, expected_time) in states.items(): time, eta, dry = read_state( path=path, record=record, expected_time=expected_time, ne=ne, nn=nn, ) if rep_eid in set(cv.tolist()): print_element_state_diagnostic( eid=rep_eid, elems=elems, depth=depth, area=area, eta=eta, dry=dry, label=name, ) V, nwet, ndry = main_cv_volume( cv=cv, elems=elems, depth=depth, area=area, eta=eta, dry=dry, ) results[name] = V print(f"{name}: time = {time:.0f} s") print(f" wet elements = {nwet:,}") print(f" dry elements = {ndry:,}") print(f" Main-CV volume = {V:,.3f} m3") print() dV = results["T1"] - results["T0"] results["DeltaV"] = dV print("=" * 72) print("PIONEER RIVER WATER LHS-1") print("=" * 72) print(f"hgrid title = {title}") print(f"Main-CV elements = {len(cv):,}") print(f"Main-CV area = {np.sum(area[cv]):,.6f} m2") print(f"V(T0) = {results['T0']:,.6f} m3") print(f"V(T1) = {results['T1']:,.6f} m3") print(f"DeltaV = V1 - V0 = {dV:+,.6f} m3") print("=" * 72) return results if __name__ == "__main__": calculate_lhs1( hgrid_path=HGRID, param_path=PARAM, cv_file=CV_FILE, states=STATES, rep_eid=149, )

# 4. Numerical LHS-1 result

Table 9. Final Main-CV water inventories and 30-day LHS-1 storage change

| Quantity | Result | Meaning |
| --- | --- | --- |
| V_CV(T0) | 5,255,103.915 m³ | Main-CV water inventory at 900 s |
| V_CV(T1) | 12,476,396.825 m³ | Main-CV water inventory at 2,592,000 s |
| ΔV = V_CV(T1) - V_CV(T0) | +7,221,292.911 m³ | 30-day increase in Main-CV water storage |

Source: Calculated from hgrid.gr3, cv_pioneer_nested_M4_N1_elements.txt, out2d_1.nc record 0 and out2d_5.nc record 191 using Eqs. (2)-(8), followed by Eq. (1).

Result interpretation. LHS-1 = +7,221,292.911 m³. The positive sign means that the Main control volume stores more water at T1 than at T0. Relative to the previous local-area calculation, ΔV increases by +20,042.470 m³ (+0.28%).